INGEO –ENVILAB, s.r.o.								SNAS		1/2

Division of Chemistry and Microbiology						Reg.No. 103/S-008

Bytčická 16

010 01 Žilina

Tel:  041/724367

Test Protocol No.: 8578/2013

- Tests ordered by:

Name of organisation: 	INGEO a.s.

Address of organisation: 	Bytčická 16, 010 01 Žilina

Company ID:		3156 2795

- Order No.		L13/027

- Kind of taken sample: water

- Reasons of taking the water sample and its analysis: Regulation of Government No. 354/2006, and  496/2010 – overall analysis of drinking water for mass supply

- Data about the examined sample:

Place of taking the sample: Turie		Sample taken by:  the order party

Mark of source:  HTU-1

Reg. No. of sample: 8578/2013			 Date of taking the sample: 09.10.2013

Date of taking over the sample: 09.10.2013

- Results of tests:

| Name of test | Value | Unit | Uncertainty U | Used method | Type of test | Limit | Evaluation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Escherichia coli | 0 | KTJ/10ml | 15% | STN ISO 9308-1 | A | 0 | conforms |
| Coliform bacteria | 0 | KTJ/10ml | 10% | STN ISO 9308-1 | A | 0 | conforms |
| Enterococci | 0 | KTJ/10ml | 16% | STN ISO 7899-2 | A | 0 | conforms |
| Pseudomonas aeruginosa | 0 | KTJ/100ml | 15% | STN EN ISO 162 | A | 0 | conforms |
| Cultiv. microorg. at 22° C | 4 | KTJ/1ml | 20% | STN EN ISO 622 | A | 2X102 | conforms |
| Cultiv. microorg. at 36° C | 0 | KTJ/1ml | 25% | STN EN ISO 622 | A | 20 | conforms |
| Colourless flagellated protozoans | 0 | individuals/ml |  | STN 75 7711 | A | 0 | conforms |
| Live organisms | 0 | individuals/ml |  | STN 75 7711 | A | 0 | conforms |
| Filamentous bacteria | 0 | individuals/ml |  | STN 75 7711 | A | 0 | conforms |
| Micromycetes determ. microscop. | 0 | individuals/ml |  | STN 75 7711 | A | 0 | conforms |
| Dead organisms | 0 | individuals/ml |  | STN 75 7711 | A | 30 | conforms |
| Iron and manganese bacteria | 0 | coverage of field % |  | STN 75 7711 | A | 10 | conforms |
| Abioseston | 1 | coverage of field % | 13% | STN 75 7712 | A | 10 | conforms |
| Clostridium perfringens | 0 | KTJ/100ML | 25% | STN EN 26461-2 | A | 0 | conforms |
| Nitrates | 12.8 | mg/l | 10% | PP-DCH-24 | A | 50 | conforms |
| Nitrites | <0.01 | mg/l |  | PP-DCH-25 | A | 0.5 | conforms |
| Fluorides | <0.10 | mg/l |  | PP-DCH-27 | A | 1,5 | conforms |
| Ammonium ions | <0.02 | mg/l |  | PP-DCH-02 | A | 0.5 | conforms |
| Aluminium | 0.043 | mg/l | 20% | PP-DCH-58 | A | 0.2 | conforms |
| ChSK-Mn | <0.05 | mg/l |  | PP-DCH-21 | A | 3 | conforms |
| Chlorides | 7.45 | mg/l | 4% | PP-DCH-20 | A | 250 | conforms |
| Manganese | 0.003 | mg/l | 10% | PP-DCH-58 | A | 0.05 | conforms |
| Ph | 7.54 | - | 1% | PP-DCH-16 | A | 6.5-9.5 | conforms |
| Sulphates | 53.5 | mg/l | 10% | PP-DCH-19 | A | 250 | conforms |
| Temperature of water | 10.6 | °C | 0.1 | PP-DCH-31 | N | 8-12 |  |
| Iron dissolved | <0.005 | mg/l |  | PP-DCH-58 | A | 0.2 | conforms |
| Electrolytic conduction | 63.0 | mS/m | 10% | PP-DCH-22 | A | 125 | conforms |
| Sodium | 2.2 | mg/l | 7% | PP-DCH-37 | A | 200 | conforms |
| Magnesium | 40.4 | mg/l | 4% | PP-DCH-10 | A | 125 |  |
| Calcium | 80.2 | mg/l | 4% | PP-DCH-09 | A | min.30 |  |
| Pathogenic microorganisms | negat. | KTJ/gl |  | STN 56 0100 | A |  |  |
| Hydrogencarbonates | 348 | mg/l | 3% | PP-DCH-23 | A |  |  |
| Free CO2 | 8.8 | mg/l | 5% | PP-DCH-75 | N |  |  |
| Mineralisation | 554 | mg/l |  | calculation | N |  |  |

INGEO –ENVILAB, s.r.o.			Test Protocol No.: 8578/013		SNAS		2/2

| Name of test | Value | Unit | Uncertainty U | Used method | Type of test | Limit | Evaluation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Silica sol ( H4SiO4) | 9.1 | mg/l | 6% | PP-DCH-77 | A |  |  |
| Baryum | 0.06 | mg/l | 15% | PP-DCH-58 | A |  |  |
| Potassium | 0.6 | mg/l | 10% | PP-DCH-35 | A |  |  |
| Lithium | 0.003 | mg/l | 15% | PP-DCH-58 | A |  |  |

Explanatory notes:  A – accredited test, N – non-accredited test, S – test carried out in the form of subcontract,

U- extended uncertainty defines the  interval  around the result of measurement  with respect to  which it is presupposed that it comprises a large share of values from division that can be attributed to a measured  value. It is calculated by multiplication of combined standard uncertainty with coefficient of coverage K =2.

SHVO  - guiding value  for taking measures.

NPHOR – the highest permissible value of radionuclide.

The stated results relate to the delivered sample. The Test Protocol may be only copied if complete, and no its part may be used without the previous approval of laboratory for promotion or publication purposes.

- Other information

Protocol worked out by:  Moravčíková Janka

Person responsible for the correctness of Protocol:  Ing. Vladimír Doboš

Date of Tests:  09.10.2013-11.10.2013		Date of the issue of Protocol: 14.10.2013

The Protocol approved by: Ing. Miroslav Záhon, director of the Division of Chemistry and Microbiology

Signature: illegible

Seal: INGEO - ENVILAB, s.r.o. Žilina , The laboratory is accredited by  SNAS for testing, certification and

accreditation S 008

……………………………………………………………………… end of protocol …………………………………………………………………